A

AB Valve

The operating device used on freight cars for charging, applying, and releasing the brakes. Also called a

triple valve

ABD Valve

An improvement of the AB Valve that features a quick release.

ABDW Valve

An improvement of the ABD Valve. Modifies the Emergency Portion and provides for accelerated buildup

of brake cylinder pressure during quick service applications.

Absolute Block

A length of track in which no train or engine is permitted to enter while it is occupied by another train or

engine.

Absolute Permissive Block (APB)

A designated section of track or tracks within which the movement of trains will be governed by block

signals, whose indications supersede the superiority of trains. The block signals may be controlled

manually or automatically.

Absolute Signal

A block or interlocking signal designated by an "A" market or the absence of a number plate.

Add, to

Couple car(s) to a train

Adhesion Coefficient

The ratio of tangential and normal force that exist between the wheel and the rail during motion.

Air Brake System

All of the devices and parts included in making an air brake for controlling the speed and stopping a

locomotive or train. It is made up of the operating devices, the pipes, fittings and foundation brake gear.

Air Test

The act of operating the brake valve to determine that the air brake system was operating correctly and

could stop the train if necessary.

Alley

A clear track in a switching yard.

Angle Cock

An appliance used for the purpose of opening or closing brake pipe on ends of cars, rear ends of tenders,

and front ends of switch engines so equipped. Provision is made for supporting hose at proper angle.

APE

All Purpose Employee -- an employee that is a promoted engineer that can also be forced to work as a

conductor or trainman.

Application

Consists of all of the operations from the time the brake pipe reduction is started until the brake is

released.

Approach Signal

A signal that governs the approach to another signal.

Articulated [Mallet]

A Mallet locomotive. A simple articulated is a mallet which had a large enough boiler to supply all four

cylinders with high pressure steam direct from the boiler. A compound mallet is a mallet which had a boiler

too small to supply high pressure steam to all four cylinders at once, and used steam twice, once to the

rear high pressure cylinders and the "partially used" steam would then supply the front cylinders. The

best known example of a compound mallet is the N&W Y6b mallet, which "shifted" to compound

operation at higher speed. Some well known simple articulated's are the UP BIG Boy, the UP Challenger,

the N&W Class A, the B&O EM-1 type, and the SP AC class.

ARU

American Railway Union, Crushed during the Pullman strike in 1894

Automatic Block Signal System (ABS)

A series of consecutive blocks governed by block signals, cab signals or both, actuated by a train, engine

or by certain conditions affecting the use of a block.

Automatic Cab Signal System (ACS)

A system which provides for the automatic operation of the cab signals and cab warning whistle.

Automatic Stop Arm

Mechanical arm located on the wayside, in conjunction with a wayside signal, which causes an emergency

brake application when a train passes the signal at danger and the arm is in tripping position.

Automatic Train Stop System (ATS)

A system actuated by wayside inductors, so arranged that its operation will automatically result in the

application of the brakes until the train has been brought to a stop.

B

Baby Lifter

A brakeman.

Bad Order

A piece of rolling stock that needs repair.

Bail

By moving the independent brake handle sideways, the engineer can release locomotive brake cylinder

pressure that is due to an automatic brake application (a brake pipe pressure reduction). The bail has no

effect on brake cylinder pressure that is due to an independent brake application.

Bakehead

Fireman (because his head was near the door of firebox when shoveling coal)

Ball (of a Rail)

The head of the rail

Bend the Iron, To

To throw a switch.

Beyer-Garratt

An articulated steam locomotive with a central boiler/cab assembly pivoted between two power units.

Designed for hard roads with tight curves, this type is found mostly in Africa.

Big Boy

Union Pacific 4-8-8-4 600 ton steam freight locomotive

Big C

The conductor (from the Order of Railway Conductors)

Big E

A railroad engineer (for Brotherhood of Locomotive Engineers)

Bighole

Emergency application of air brakes, usually when initiated by engineer, i.e. put her in the big hole

BIE

Brakes In Emergency; application of the emergency braking system.

BLE

Initials of Brotherhood of Locomotive Engineers union

BLF&E

Initials of Brotherhood of Locomotive Firemen and Engine-men union

Blinds

A walk way between two passenger cars covered with either canvas or leather in an accordion shape.

From the outside of the blinds to the outer edge of the cars there was a space about 24 inches wide. There

was a ladder running up to the top of the car in this space and the bums would grap hold of the ladder and

hold on to it. That was riding the blinds.

Block

A length of track between consecutive block signals or from a block signal to the end of block system

limits, governed by block signals, cab signals or both.

Block Occupancy Indicator

An indicator used to convey information regarding block occupancy.

Block Signal

A fixed signal at the entrance of a block to govern trains and engines entering and using that block.

Block System

A block or series of consecutive blocks within APB, ABS, ACS, CTC or interlocking limits.

Block Track

Track with equipment for repairing rail cars on the spot.

Blue Flag

A blue flag or signal that is placed on a car or locomotive when workers are around or under it. When a car

or locomotive is blue-flagged, then it must not be coupled to or moved in any manner. The only person

allowed to remove a blue flag is the person who put it there in the first place.

BMT

Brooklyn Manhattan Transit - subdivision B-1 of the New York City Transit Authority (NYCTA) subway

Bobby

British: term for a signalman in UK. Derives from railway policeman of early railways. The policeman being

'invented' by Sir Robert (Bobby) Peel.

Boomer

Itenerent railroad workers. Always moving from one road to another.

Booster

A small two cylinder steam driven engine, manufactured by the Franklin Railway Supply Co., and attached

to an axle of the trailing track of some steam locomotives to provide additional tractive effort when starting

a train.

Bradley Bar

A device shaped like a hockey stick used to straighten hand holds on freight cars.

The Brains

The conductor

Brake Beam

A cross-piece in the foundation brake gear for a pair of wheels to which the leverage delivers its force to

be transmitted through the attached brake head and brake shoes to the tread of the wheels.

Brake Cylinder

A cast metal cylinder with a piston that is forced outward by compressed air in applying the brakes and

returned by a release spring in releasing the brakes.

Brake Pipe

Commonly called a train line, it is the pipe, hose, connections, angle cocks, cut-out cocks, fittings, etc.,

connecting the locomotive and all cars from one end of the train to the other for the passage of air to

charge and control the brakes.

Brake Rigging

A term commonly used instead of foundation brake gear.

Brakes, Automatic

Automatic brakes are the brake controls in the locomotive that regulate the pressure of the brake pipe and

apply or release the brakes for the entire train including the locomotives

Brakes, Independent

Independent brakes are the brake controls in the locomotive that apply the brakes on the locomotives

only. The air hose marked ACT or BR CYL enables the lead unit to control the trailing units brakes

Branch

A portion of a division designated by a timetable. Rules and instructions pertaining to subdivisions apply

on branches.

Branch Line

A secondary line of a railroad, not the main line.

Brass Hat

AA railroad executive, usually a division manager or higher, a.k.a. suits

Bridge Line Haul Road

See overhead line haul road.

Brotherhood Notch

A notch high on the reverse lever quadrant which admitted a very limited amount of steam to the cylinder

making it easier on fireman, but taking longer to get over the road.

BRT

Initials of Brotherhood of Railway Trainmen union

Bull

Slang for a railroad police officer or railroad detective.

Bullhead

A condition where both drawbar knuckles are closed, making the coupling impossible without opening

one knuckle (amer. slang).

C

Cab Apron

The hinged metal plate attached to the rear end cab floor of a steam locomotive, which rested on the front

of the tender and formed a transition piece between the engine and tender for crew safety. Also called a

deck plate.

Cab Forward

A steam locomotive with the engineers cab placed ahead of the boiler instead of behind it.

Centipede Tender

A high capacity tender applied to some large steam locomotives, and having seven axles, with the front

two axles contained in a track casting and capable of swiveling. Major users of centipede tenders include

New York Central and Union Pacific railroads (USA).

Cab Signal

A signal located in engineer's compartment or cab, indicating a condition affecting the movement of a train

or engine and used in conjunction with interlocking signals and in conjunction with or in lieu of block

signals.

Cabin Car or Hack

A non revenue car formally used on the rear of trains (caboose)

Caboose

End of train non revenue car

Caboose Valve

A rotary valve type of device providing means for making a controlled rate of brake pipe reduction for

making a service or emergency application from the caboose.

Cantrail

From the 1889 Century Dictionary of the English Language: A timber running along the tops of the

upright pieces in the sides of the body of a railway-carriage and supporting the roof and the roof sticks.

Called in the United States a plate.

Car Knocker

It was common for car inspectors to tap parts with a hammer. The resulting tone of sound gave clues to

the condition of those parts. Ergo, Car Knocker.

Carman

Formal name for a craft employee that inspects and repairs railway cars.

Car Toad, Car Tonk

Car inspector who checked the condition f freight and passenger cars and conducted the air brake tests

CCR

Central Control Room, a facility from which rail system operation will be monitored and controlled.

Centralized Traffic Control (CTC)

A remotely controlled block signal system under which train movements are authorized by block signals

whose indicators supersede the superiority of trains.

Cinder Dick

Railroad detective (slang)

CLC

Complete Locomotive Control, retrofitted adhesion system manufactured by Woodward Governor

Company.

Clear Block

A block not occupied. Sometimes used to denote a clear signal indication.

COFC

Container on flat car. Referred to in intermodal traffic.

Color Light Signal

A fixed signal in which the indications are given by the color of a light only.

Color-Position Light Signal

A fixed signal in which the indications are given by color and position of two or more lights.

Company Notch

Denotes reverse lever in lower quadrant using lots of steam to pull a heavy, i.e. revenue train making

money for the company.

Conductor

Brakeman, with or without brains, displaying pencils.

Controlled Point

A location designated by number where signals and /or switches of a CTC system are controlled by a

control operator.

Controlled Siding

A siding within CTC or interlocking limits, the authorization for use of which is governed by signal

indication or control operator.

Controlled Signal

An absolute signal, the aspect of which, is controlled by a control operator.

COT&S

Clean, Oil, Test & Stencil. Applies to air brake rework.

Covered Wagon

A nickname that is generally attached to EMD E and F units.

Creeper

A rail anchor of spring steel that is driven onto the base of the rail and bears against the tie which is

prevented from moving by the resistance of the stone ballast. The name comes from the function of the

anchor which is a "rail anti-creep device."

Crossing

A length of track that carries one track across another.

Crossover

A track connection between two adjacent tracks.

Crummy

A wooden, two truck or bobber trucked, caboose. Also called a way car, hack or, in the days of living in

them, a bean shack

Cupola

A small cabin on the roof of a caboose to afford a means of lookout for the train crew.

Current of Traffic

The movement of trains on a main track, in one direction, specified by the rules.

Cut, to

Separate car(s) from a train

Cut Lever

The hand operated lever applied to all cars and locomotives, which was used to lift the coupler pin and

release the knuckle in order to couple or uncouple cars and locomotives.

D

Dark Territory

A series of rail miles ungoverned by signals and unable to transmit or receive radio or cellular phone

signals.

Date Nail

A small nail used by railroads from late 1800's to present used to mark the year a tie was placed in roadbed.

Nails are distinctive in that each has the last two digits of placement year stamped in head. Usually found

within six inches of tie end, but some are located mid tie to allow easier inspection. Rarer nails value in

100's of dollar range to collectors

Dead Head

A railroad employee traveling on a pass.

Dead Man

Usually a foot pedal that was pressed by the engineer. When pressure on the pedal was released, the train

brakes were automatically applied. This was to detect sleeping or dead engineers.

Deckorate

In the days before air brakes, the duties of the brakemen included stopping the train. The brakeman would

have to go to the top deck of the car - thus decorate - and wind the stem winder.

Derail

A device placed short of clearing point on a track to prevent a car or engine from fouling main track,

derailing said car or engine if not removed to permit safe passage.

Diamond

A special track work item that allows two railroad tracks to cross each other at grade.

Dinger

A yard master.

Distant Signal

A fixed signal outside of a block system, used to govern the approach to a block signal, interlocking signal

or switch point indicator. It will not convey information as to conditions affecting the use of the track

between the distant signal and block signal, interlocking signal or switch point indicator to which

approach is governed. It will be identified by a "D" marker.

Division

A portion of the railroad designated by timetable.

Dogcock

A device used in unison with a clawbar to pull spikes from the wing rails of a frog and also from the guard

rail.

Dollyflopper

A brakeman or switch-tender - someone who throws switches.

Doorslammer

Slang for a passenger trainman. Usually used by freight trainmen who are adept at station switching, and

all the other skills needed in general freight service.

Double Slip Switch

Used only where space is limited, combines the functions of a crossing and turnouts to allow any one of

four routings.

Double Track (DT)

Two main tracks, on one of which the current of traffic is in a specified direction, and on the other in the

opposite direction.

Drag

A common expression to describe the movement of a heavy train, such as a coal drag or an ore drag.

Drawbar Horsepower

The total horsepower of a locomotive less the amount of horsepower that it takes to move the locomotive

itself, the balance being available to pull the load.

Drill Track

A track connecting with the ladder track, over which locomotives and cars move back and forth in

switching.

Drop

Switch a car behind the engine onto an adjacent track when the engine can't run around the car. Requires

two trainmen, one to pull the pin on the car to be dropped and the other to throw the switch after the

engine has passed to let the car run onto the parallel track.

Dual Control Switch

A power-operated switch, also equipped for hand operation.

Dummy

A small auxiliary signal used to control unusual movements such as a set back into a yard from a main line.

Implies a complete stop and wait for a manual operation from the panel. Usually ground mounted lens: two

whites for proceed and red/white for stop. Also known as dolly or dwarf.

Dump the air

Emergency application of the air brakes causing a train to stop abruptly, usually causing damage to the

merchandise being carried or to the train equipment, itself

Dutchman

A short section of brake hose with a coupling(glad hand) on each end. It's used to connect two short

hoses together.

Dwarf Signal

Two or three lens signal used to control a move over a switch in a yard.

Dynamic Braking

A method of train braking where the kinetic energy from the train movement generates current at the

locomotive traction motors, and is dissipated in a resistor grid on the locomotive.

Dynamite

Initiation of an emergency application.

Dynamiter

A term commonly given to an brake operating valve that goes into quick-action emergency when it should

not. Also called a Kicker. Dynamiting -- application of emergency (air) brakes.

E

Electric Switch Lock

An electrically controlled lock device affixed to a hand operated switch or derail to control it's use.

Electric Switch Lock

An electrically controlled lock device affixed to a hand operated switch or derail to control it's use.

Elephant Ears

Metal side plates used on some large steam locomotives to lift the smoke above the train at speed.

Emergency Application

An application resulting from an emergency rate of brake pipe reduction which causes the brakes to apply

quickly and with maximum braking force for the shortest practical stopping distance.

Engine

A unit propelled by any form of energy, or a combination of such units operated from a single control,

used in train or yard service.

Engine Lite

Locomotive or multiple units lite of any cars.

Engine Whistle Signals

* means a short blast of the whistle or horn

- means one long blast

*

apply brakes, stop

* *

answer to any signal not otherwise provided for

* * *

when standing, back

* * * *

call for signals

-

test train brakes

- -

release train brakes

- - -

when running, stop at next passenger station

- - -

when standing, train parted

- - - -

recall flagman from south or west

- - - - -

recall flagman from north or east

- * *

calling attention to another train that signals are displayed for a following section

- * * *

flagman protect the rear of train

* * * -

flagman protect the front of train

- - *

approaching meeting or waiting points

- - * -

approaching crossing at grade

- * * -

answer to yellow temporary reduced speed flag placed 1 1/2 miles in advance of

restricted tracks

EOT

End Of Train unit (see also Caboose). An EOT transmits brake pipe pressure to the lead unit (head end

locomotive), while a two way EOT is also capable of receiving a transmission from the lead unit to open the

brake pipe and put the train into emergency stop (clarified by Bob Murphy).

Extra Train

A train not authorized by timetable schedule. It may be designated:

Extra -

For any extra train except work extra, the movement of which is authorized in a specified direction.

Work Extra -

For any extra train authorized by Form H train order, the movement of which may be in either

direction within specified limits.

F

Facing Point Lock

A locking device which automatically locks the switch points of a spring switch in normal position.

Fairlie

Double ended Locomotive with a single central cab, Designed by Robert Francis Fairlie. Always running

cab forward. See http://www.bangor.ac.uk/ml/fr/fairlie.jpg. Robert Francis Fairlie also designed a 'single'

locomotive with one powered and one unpowered boogie and with a conventional cab at one end so can

travel chimney first or bunker first.

Ferro-Equinologist

Ferro- meaning iron plus equine- meaning horse give one who studies iron horses, i.e., a railfan.

Fire Box

The "stove" where the wood, coal, oil, etc., was burned to make steam to propel the engine.

Fishplate

Length of iron, applied to either side of rail web, used to connect sections of rail together.

Fixed Signal

A signal of fixed location indicating a condition affecting the movement of a train.

Flagman

The rear brakeman. The great country music singer Jimmie Rodgers used to brag about being a flagman.

Reason? Because flagmen had to know how to read so they could understand train orders.

"Flat"

A slang term use by graffiti writers for a box car without ridges on the the sides. Perfect for applying illegal

graffiti by the medium of spray paint.

Flimsy

Train order, or paper paper used for train orders.

Flying Duck

A derogatory term used to describe a switchman of the former Pennsylvania Railroad, who customarily

gave hand signals to their enginemen using both hands at once.

Flying Shunt

A method to roll a car into a stub track when the train is approaching from the opposite side of the switch

to the stub track.

The train is stopped several yards from the switch. The engine and the car to be dropped is uncoupled

from the main part of the train, and the brake reservoir on the car to be dropped is emptied. Brakeman #1

rides the car to be dropped, and brakeman #2 operates the switch.

The engine is accelerated, just prior to reaching the switch the engineer slacks the throttle, brakeman #1

pulls the uncoupling handle, then the engine speeds up, pulling away from the rolling car. Once the engine

passes the switch, brakeman #2 throws the switch allowing the rolling car to go in the stub track.

Once the car is by the switch brakeman #1 applies the hand brake to stop the car. The engine now can be

backed up and then used to spot the car.

Although this action was considered to be unsafe, it was occasionally done.

Forestalling Lever

A lever next to the engineer's position on locomotives used by railroads with Intermittent Inductive Train

Control. This control system would cause an automatic brake application if an engineer violated a

restrictive signal, and the system required that the engineer operate the lever (ie. forestall) when passing

each signal to prevent air brake automatic application which would stop the train. The IITS system

included a magnetic shoe signal pickup mechanism, which was placed on the first tender axle on steam

locomotives and on the lead axle on diesel locomotives, and wayside inductors in each signal block. This

system was in use for many years on the New York Central railroad.

Form D

A form used in receiving written permission to occupy track in DCS sections of railroad lines. Permission

is given by Train Dispatcher or Operator.

Forty Five

Yellow signal or semaphore at 45 degrees. Train may proceed through signal, prepared to stop.

FRED

Flashing Rear End Device -- end of train telemetry device

Frog

The intersection of two rails of a switch.

Front End

A term used to describe the smokebox end of a steam locomotive, including the exhaust stack, netting, etc.

Full Service Application

Corresponds to a handle position for the automatic brake handle. In this position the brake pipe should be

at 62 PSI (down from a 90 PSI release charge pressure). When an application is made on the automatic

brakes, the equalizing reservoir pressure drops in proportion to the handle movement. The self lapping

valve (Automatic Brake Valve) then vents brake pipe pressure at a service rate until the equalizing

reservoir and brake pipe pressures are equal. This pressure is measured on the locomotive only. It may be

less further back on the train due to leakage. A minimum reduction is a 6 PSI drop to 84 PSI. After a

minimum reduction is made, the automatic brake valve handle is linear down to zero. If the locomotive has

a direction on the reverser handle, or the independent brakes are released, below 45 PSI BPP an emergency

will occur and a valve will blow the brake pipe to zero in a hurry (corrected by Bob Murphy).

Fusee

A warning device consisting of a cardboard tube filled with a combustible mixture of chemicals that burns

brightly when ignited and remains burning for varying lengths of time. Fusees are ignited and dropped on

the right of way to indicate to a following train the presence of stopped or slow-moving equipment ahead.

G

Gandy Dancer

A railroad track worker. Name came from the Gandy Mfg Co. in the 19th century that made a lot of track

tools.

Gateway

See Interchange Point

Gauge

Broad gauge (Spain):

1674 mm

5'5 9/10th"

Broad gauge (Portugal):

1665 mm

5'5 11/20th"

Broad gauge (Ireland):

1600 mm

5'3"

Broad gauge (Finland):

1524 mm

5' exactly

Broad gauge (former USSR):

1520 mm

5'

Standard gauge:

1435 mm

4'8 1/2"

Narrow gauge (Cape gauge):

1067 mm

3'6"

Narrow gauge (meter gauge):

1000 mm

3'3 37/100"

See also a more or less complete list of railroad gauges.

Glad hand

The metal attachments to which train line air hoses connect

Goat

A yard engine.

Groundhog

A slang term for a promoted engineer with trainman seniority.

Grade Resistance

Resistance that results from the energy you must put into a train to lift it vertically. The energy is returned

without loss when the train comes back down again.

Grainer

Rail slang for covered hoppers, which are often used to transport grain and other bulk, fluid solids.

Green Eye

A slang term for a clear signal.

Grip

Trainman's suitcase.

H

Haul, Short

The act of routing freight such that the haul takes maximum advantage of the originating railroad, at the

disadvantage of another railroad which had to be used to carry the freight part of the way to its

destination. The railroad which suffered the disadvantage was said to be "short hauled."

Head End

The front of the train. Use of this term is declining with the demise of the caboose.

Head End Power

A power system installed on diesel-electric passenger locomotives, used to generate "hotel load" power

to the passenger train, including train heating and air conditioning. A head end power system may use

either the locomotive's prime mover, or it may use a separate HEP engine generator set installed at the #2

end of the locomotive.

Helper

Any locomotive added to assist a train up a grade.

Highball

A signal given to proceed at maximum permissible speed.

High Iron

The railroad's mainline, usually with more ballast and heavier rail, which made this track higher than yard

track.

High Rail

Main track.

Hobo

An individual who rides freight trains to get from town to town. Not to be confused with a bum, a hobo is

a transient worker.

Hog

A locomotive

Hoghead, Hogger

A railroad engineer

Hog Law

Refers to ICC hours of service regulations.

Holding Lights

Amber or green light signal displayed at certain station platforms at or near the conductor's position, to

regulate train spacing.

Holy Roller

A graffiti slang term for a car transport car. Like for their great length, perfect for doing an end to end and

other large "productions" with the illegal spray-paint techniques.

Home Signal

A special red signal that requires the train crew to call the dispatcher for orders before the train can

proceed.

Hooking Up

The act of shortening the duration of the steam admission setting on a steam locomotive, using a Johnson

Bar or Power reverse wheel or lever, thereby trading power for speed.

Horsepower per Trailing Ton.

The total horsepower of all working locomotives divided by the total trailing weight of the train in tons.

Horsing Lever

The lever on a steam locomotive used to manually adjust the valve setting (ie. cutoff). Also known as the

Johnson bar. When engines became larger in size, a manual adjustment was no longer practical and air

operated motors were used for this purpose, and a smaller lever or wheel located in the cab of the steam

locomotive was used to adjust direction and cutoff.

Hostler

A person who operates engines in engine house territory and works under the direction of the engine

house foreman (inside hostler). Some railroads created outside hostlers after a limited exam, who could

deliver engines anywhere in the terminal.

Hostler's Controls

A simple throttle to allow independent movement of locomotives not equipped with engineers controls.

Hot Box

On friction bearings, an overheated journal bearing.

House Track

A track entering, or along side a freight house. Cars are spotted here for loading or unloading.

I

IND

INDependent City Subway - subdivision B-2 of the New York City Transit Authority (NYCTA) subway

Initial Station

The first station on each subdivision from which a train is authorized to occupy the main track.

Interchange Point

The point at which two or more railroads join. Traffic is passed from one road to another at interchange

points.

Interlocking

An arrangement of signal appliances so interconnected that their movements must succeed each other in

proper sequence. It may be operated manually or automatically.

Interlocking Limits

The tracks between the outer opposing absolute signals of an interlocking.

Interlocking Signals

The fixed signals of an interlocking, governing trains using interlocking limits.

Intermodal

Freight traffic that refers to containerization of freight for easy transloading to different modes of

transportation. See TOFC,COFC, Piggyback.

Iron Bender

A switchman.

IRT

Interboro Rapid Transit - subdivision A of the New York City Transit Authority (NYCTA) subway

J

Jerkwater Town

A small town with few facilities, identified on the railroad by the existence of a water plug only.

Jew Bar

A device that holds a track in gauge. Used on sidings or industrial track.

Johnson Bar

Valve gear adjustment lever.

Joint Facilities

Any facilities owned by two or more railroads.

Journal Box

Metal box around axle bearing for holding a lubricant saturated pad next to the wheel bearing.

Jumper

British: a passenger leaping from a moving train on the blind side to avoid paying.

K

Kettle

(Steam) engine (amer. slang).

Key-By

The act of lowering an automatic stop arm in order to pass a red signal.

Kicker

A common expression for an emergency brake application which occurs when a service brake application

is intended or when no application is intended.

Knuckle

The movable portion of the drawbar coupler.

L

Ladder Track

A series of turnouts providing access to any of several parallel yard tracks.

Layshaft

A hand operated throttle connected to the governer on a diesel locomotive.

Lead Rail

Rail between the frog and the switch.

Line Haul Road

A railroad that handles freight over a medium to long distance.

Locomotive

Locomotives are units propelled by any form of energy, or a combination of such units operated from a

single control station, used in train or yard service

Locomotive Speed Limiter

A modern device used to control train speeds. All engines on Amtrak's NEC must be so equipped.

Low Arm

A nickname for a restricting signal in the days of the semaphore with the arm down 45 degrees.

Low Irish

Stands for medium clear signal.

Lubricating Arm

The lubricator forced valve oil into the valve chamber to lubricate the steam valve, and the lubricating arm

on a mechanical lubricator drove the pump that supplied this lube.

M

Main Track

A track extending through yards and between stations which must not be occupied without authority or

protection.

Mallet

Reference to the Mallet Articulated Cab Forward steam locomotives used by Southern Pacific railroad in

the 30's, 40's and 50's.

Manual Block System

A series of consecutive blocks, governed by block signals operated manually, upon information by

telegraph, telephone or other means of communication.

Maplines

Short lines in Connecticut and Massachussets, USA

Marker

A train signal that is used to indicate the end of the train.

Mikado

A steam freight locomotive having a 2-8-2 wheel arrangement, pioneered in a design for the emperor of

Japan in 1912.

Mile Post

A post or sign on pole each mile along the track that shows the distance from a predefined location such

as a major rail terminal.

Monkey Motion

Slang for the valve gear linkage on a steam locomotive.

Monkey's Tail

Slang for the handle of a switch stand, as in twisting the monkey's tail.

Motor

1.The electrical machine (traction motor) geared to the axles of all diesel-electric and electric

locomotives, and used to convert the electrical energy provided by the diesel engine and main

alternator in a diesel electric locomotive, or the transformer output in an electric locomotive, to

mechanical force in the form of tractive effort.

2.The descriptive term used on the electric division of the Great Northern railroad to designate an

electric locomotive.

MOW

Maintenance Of Way

Multiple Main Tracks

Two or more main tracks, the use of which is designated in the timetable.

MU

Multiple Unit. A lead locomotive followed by one or more locomotives. Cables between the MU

connectors bring the electrical signals in party line fashion to the trailing units (clarified by Bob Murphy).

Mud Ring

The lower part of the boiler of a steam locomotive directly in front of the firebox, where boiler scale and

sediment settled as the engine operated. A removable plug was located at the bottom of the boiler in this

area, and this plug was removed during the monthly boiler wash to flush this contamination from the

boiler.

Mule

A movable, hand-carried derail that is placed on either rail.

Muzzle Loader

Term used to describe a hand fired locomotive.

N

NEC

North-Eastern Corridor -- the Amtrak route with intensive passenger traffic that connects Washington,

DC, Philadelphia, New York City, and Boston

19 & 31 train orders

These types of orders were transmitted to train crews. They covered vast area of conditions such as fixing

meeting points, speed restrictions.

NORAC

Northeast Operating Rules Advisory Committee: the operating rules the many USA Northeast Freight and

Passenger Railroads operate under.

Number Dummies

Clerks who worked as yard checkers

O

ORC

Old Reliable Conductors -- The union that represented conductors during the mid to late 19th century.

Originating Line Haul Road

The railroad where any freight shipment starts.

Originating Station

The first station on each subdivision from which a train is authorized to occupy the main track.

OTM

Other Track Material -- materials other than ties and rails, generally refers to spikes, tie plates and rail

anchors

Overhead Line Haul Road

Any railroad or railroads between the originating line haul road and the terminating line haul road. Also

known as a bridge line haul road.

Overlap Sign

A sign marking the limit of control of a block signal.

P

Paired Track

When two railroads own single track lines, they may reach an agreement whereby one railroads track

services both roads in one direction, while the other railroads track services both roads in the other

direction.

Panel

British for the Centralized Control Tower.

Partial Service Application

Reducing the brake pipe pressure at a service rate but not enough to cause the reservoir and cylinder

pressure to equalize.

Peg

British: a signal post.

Piggyback

TOFC or trailer on a flat car. Originally used when truck trailers were loaded onto flat cars for shipment by

rail.

Piglet

A locomotive engineer trainee.

Pilot

An employee assigned to a train when the engineer or conductor is not acquainted with the rules or

portion of a railroad over which the train is to be moved.

Pin

A piece of metal used to lock the coupler to keep cars coupled together. Also a term meaning to push in

the slack when uncoupling cars.

"Pins and Knuckles"

P/K, train inspection.

Plant

Slang for interlocking.

Plug

Illegal device to continuously bail independent brake pressure. Could be simply a coin or a more

elaborate device wedged above the independent brake valve.

(verb, to plug it) To place the brake handle in emergency.

Pocket

Portion of track within a terminal on which a train may stand for a period of time

Pomptrolli

South-African: a hand-operated pump-action inspection cart.

Pony Truck

The casting and wheel set or wheel sets which make up the leading wheels of most steam locomotives,

intended to guide locomotives through curves and switches, and used to properly distribute the weight of

the locomotive.

Position Light Signal

A fixed signal in which the indications are given by the position of two or more lights.

Power

A name used to mention the engine units on a train.

Prime Mover

A V-type diesel with 8 to 20 cylinders rated at about 125 hp per cylinder if normally aspirated or 250 hp per

cylinder if Turbo charged.

Private Car/Business Car

Coaches owned by private individuals/railroad (for use of corporate officials or supervisors). Cars were

positioned at end of trains and train crew were to remain off these cars except in performance of duties.

Crew was also to see that occupants of these cars were not disturbed at all costs

Pumping Signal

Any fixed signal including Absolute and Intermediate block signals, who's indications change rapidly

from one indication to another and then back again due to track circuit or signal circuit failure. An engineer

encountering such a signal will be governed by the most restrictive indication the signal can display.

Pusher

A helper added at the rear of a train.

Pussyfoot

Railroad detective, police, or security personnel, often found in plain clothes in rail yards or piggy-back

lifts where high-dollar freight is being moved.

Puzzle Switch

Another name for a slip or double slip switch.

R

Rail Weight

The number of pounds per yard that rail weighs. Currently rail is being rolled at 112 to 145 pounds per

yard.

Rake

Wagons/carriages semi permanently joined in an articulation rather than via a coupler

Red Eye

A red signal or horizontal semaphore arm requiring the train to stop and proceed with caution.

Reefer

A common slang term for a refrigerator car.

Regenerative Braking

Braking mode of modern electric locomotives, where the motors act as generators as with the dynamic

braking but, instead of being converted into heat, the current is fed back to the supply. Return energy

from asynchronous motors is around 90%.

Register Station

A station at which a train register is located.

Regular Train

A train authorized by a timetable schedule.

Repeater Signal

Signal placed on the opposite side of the track from the controlling signal. It repeats the aspect of the

controlling signal for a greater range of vision.

Rerailer

A heavy metal casting which was designed to be placed near a derailed wheelset of a locomotive or car,

for the purpose of guiding the wheelset back onto the rail. Steam locomotives and early diesels usually

carried rerailers on hooks on the tender trucks or frame (steam locomotive) or on the frame of a diesel.

Restricted Speed

A speed that will permit stopping within one half the range of vision; short of train, engine, railroad car,

stop signal, derail or switch not properly lined, looking out for broken rail, not exceeding 20 MPH.

Retainer

A device added to the braking device on a car, to allow a portion of the air pressure to be retained in the

brake cylinder of a car to help restrict the movement of a train on severe downgrades.

Revenue Collection Train

A train which picks up the revenue collected by the railroad clerk.

Ribbon Rail

Continuous welded rail, laid in 1/4 mile lengths then welded end to end to make a continuous length.

Rip Track

A small car repair facility, often a single track in a small yard. Name derived from "Repair, Inspect and

Paint."

Rolling Resistance

Resistance that is made up of wheel friction, journal friction, and wind resistance. It is non recoverable.

Ruling Grade

The particular point on the run at which the combination of grade and curve resistance makes the train pull

hardest and , therefore, "rules" how heavy a load can be given to the locomotive.

Run In

Describes the action of the slack between the cars moving forward and hitting against the engine. A run

out would be the opposite effect.

S

Saw-by, Double Saw-by

Maneuver used by two trains at meeting point, when train on siding is too long for the siding. Double

saw-by is complicated maneuver allowing two trains that are both longer than the siding at meeting point

to pass one another at that siding.

SBU

Sense and Brake Unit (see also Caboose)

Scab

Non union member doing work usually contracted by railroads for railway union labor contracts.

Scrap Iron

Broken knuckle due to uncontrolled slack action in train or overly aggressive starting technique.

Schedule

That part of a timetable which prescribes class, direction, number and movement for a regular train.

Section

One of two or more trains running on the same schedule, displaying signals or for which signals are

displayed.

Semaphore Signal

A signal in which the day indications are given by the position of a semaphore arm.

Shay

A type of steam locomotive using a gear drive in place of a side rod drive, designed by Ephraim Shay in

the late 1800's, and produced by what became the Lima Locomotive Works. This locomotive was designed

for logging and other operations where heavy grades and sharp curves existed and prevented the use of

side rod type locomotives.

Shock

The effect of a sudden change in speed of a car, locomotive or train, or part of a train.

Shoe Fly Track

A temporary track built around a train wreck or washout

Side Track

A track auxiliary to the main track.

Siding

A track auxiliary to the main track for meeting or passing trains. The timetable will indicate stations at

which sidings are located.

Signal Aspect

The appearance of a fixed signal conveying an indication as viewed from the direction of an approaching

train; or the appearance of a cab signal conveying an indication as viewed by an observer in the cab.

Signal Dolly

Train that delivers supplies to towers.

Signal Indication

The information conveyed by the signal aspect.

Signalman

The man who controls the signals and authorizes the movements of trains on running lines

Single-Car Test Device

Is used to test the air brake equipment on car that is sent to a repair track

Single Track

A main track upon which trains are operated in both directions.

Skipper

The conductor

Slack

The motion, forward or back, that one or more cars, locomotives, or parts of a train has without moving

other coupled cars, locomotives, or parts of the train. Loose slack is the free movement or lost motion

between parts of a train. Spring slack is the movement beyond the free or lost motion brought about

through compressing the draft gear springs. Slack is necessary so as to start one car at a time and so that

the train may be operated around curves and over high and low places.

Slack Action

Movement of part of a coupled train at a different speed than another part of the same train.

Slippery Track

A highly greased track near the roundhouse or back shop where a newly rebuilt locomotive could be run

in without going anywhere, and without calling an engine crew or pilot.

Slug

A small, ballasted, four or six axle unit, semipermanently coupled to a locomotive that does not have a

prime mover, but does have traction motors. Generally used in yard duty where the switcher has enough

horsepower, but not enough tractive force to push long strings of cars up a hump.

Smoking to a Meet

In steam service, pre-radio, making smoke to alert awaiting opposing train that you were approaching

meeting point.

Snake

A switchman belonging to the SUNA, the Switchman's Union of North America.

Snipe

The title of a track laborer or Gandy dancer. One who builds or repairs railroad track.

Speeder

A four-wheel MOW vehicle to carry men and supplies to and from a railroad work site.

Spotting

The act of placing a car in a specific location on a track.

Split Switch

A term referring to the condition that exists at a switch when one pair of wheels under a car follows a

course different from all other wheels under the car, generally resulting in a derailment.

Spring Switch

A switch equipped with a spring mechanism to restore the switch points to original position after having

been trailed through.

Standing Cut

A term for making a cut of cars by walking to the cut to be made rather than pulling the cut to you

Station

A place designated in the timetable station column by name.

Stem Winder

Nickname for a staff brake which consists of a vertical rod and a wheel at the top of the rod for the

leverage to wrap the brake chain around the vertical rod to stop or secure the car. A pawl was provided to

hold the brake applied.

Stinger

A portable insulated pole used by railroads (and transit authorities) with third rail trackage. The pole is

used to "reach" from an existing third rail power source to the pickup shoes of the electric locomotive in

instances where the shoes of the locomotive are not contacting the third rail. The stinger can also be used

to move electric locomotives within a shop complex.

Stub Track

A form of side track connected to a running track at one only and protected at the other end by a bumping

post or other obstruction.

Subdivision

A portion of a division designated by timetable.

SUNA

Initials of Switchmen Union of North America

Sun Kink

A section of rail that elongates and bends out of alignment due to heat expansion.

Superior Train

A train having precedence over another train.

Swing Man

Supplementary brakemen added to a crew for all or part of a trip, perhaps to give more hand brake capacity

in mountainous territory, or for other reasons.

Switch Point Indicator

A light type indicator used in connection with facing point movement over certain switches to indicate

switch points fit properly.

T

Tallow Pot

Fireman. In the 1800's tallow was used as a lubricant.

Tangent Track

Straight track.

Tare Weight

The weight of an empty car.

TCS

Traffic Control System

TCU

Transportation Communications International Union, represents clerks, car-men, yardmasters, and

supervisors.

Team Track

A track on which rail cars are placed for the use of the public in loading or unloading freight.

Telemetry

Another name for an EOT device which transmits End Of Train info to engine. Also "Telem", "Tele",

"FRED", and "Freddie".

Tender

A vehicle connected to most steam locomotives which carried the coal (or oil) and water for the

locomotive.

Terminating line haul road

The last railroad over which any shipment travels.

Terminating Station

The last station on each subdivision to which a train is authorized to occupy the main track.

Throbbing Red

A single flashing red light indicating Stop and Proceed or in some cases (depending on the railroad) a

flashing red light indicating Restricting. No stop required, however a speed restriction applies.

Timetable

The authority for the movement of regular trains subject to the rules. It may contain classified schedules

and includes special instructions.

TOFC

Trailer on a flat car. Refers to intermodal shipments.

Tons per Operative Brake

Gross trailing tonnage of the train divided by the total number of cars having operative brakes. (not

including locomotives)

Tommy Dodd

British slang for a subsidiary semaphore signal on the same post or bracket as the main signal to which it

applies.

Torpedo

An explosive cap fastened to the top of the rail and exploded by the pressure of a rolling wheel to give an

audible indication of conditions on the track ahead.

Track Bulletin

A notice containing information as to track conditions or other conditions, necessary for the safe

operation of trains or engines.

Track Car

Equipment, not classified as an engine, which is operated on track for inspection or maintenance. It may

not shunt track circuits or operate signals and will be governed by rules and special instructions for trains

other than passenger trains.

Track Circuit

An electrical circuit of which the rails of the track form a part. The track circuit is the basis of signaling

systems.

Track Gauge

The distance between the inner faces of the track heads. Nominally, 4' 8.5".

Track Head

The top of the track on which the wheels roll.

Track Pan

A water filled trough placed between the rails at certain locations on a railroad's main line, each trough

having a length of up to 2500 feet, for the purpose of adding water to the tender of a steam locomotive via

an air activated scoop which was located on the underside of a locomotive tender. The use of a track pan

arrangement prevented a need to stop to obtain water. Users of track pans included the New York Central,

the Pennsylvania, and the Baltimore and Ohio Railroads in the US.

Track Permit

A form used to authorize occupancy of main track where designated by special instructions.

Track Side Warning Detector

Wayside detectors which are provided at various locations as shown in the timetable which detect such

conditions as overheated journals, dragging equipment, excess dimensions, shifted loads, high water and

slides.

Track Warrant Control (TWC)

A method of authorizing movements of trains or engines or protecting men or machines on a main track

within specified limits in territory designated by special instructions or general order.

Track Web

The thin section of track between the base and the head.

Trackage Rights

An agreement between two railroads according to which, one railroad buys the right to run its trains on

the tracks of the other, and usually pays a toll for the privilege. That toll is called a "wheelage" charge.

Tractive Force

The amount of force at the driving wheel rims to start and move tonnage up various grades.

Trailing Truck

A fabricated or steel casting containing one, two, or three wheel sets, located under the engine cab and

firebox of some steam locomotives.

Train

An engine or more than one engine coupled, with or without cars, displaying a marker and authorized to

operate on a main track.

Train Brake

The combined brakes on locomotive and cars that provides the means of controlling the speed and

stopping of the entire train.

Train Line

A cable of series of hoses used for connecting electrical or steam (in older passenger equipment).

Train of Superior Right

A train given precedence by train order.

Train of Superior Class

A train given precedence by time table.

Train of Superior Direction

A train given precedence in the direction specified in the time table as between opposing trains of the

same class.

Train Order

A message changing the meeting point between two trains. For movement of trains not provided by

timetable train orders will be authorized by, and over the signature of the director of train dispatching or

chief dispatcher.

Train Order Signal

Fixed signal near the entrance to a river tube, bridge or at stations with moving platforms. Two lunar white

mean Proceed without orders according to rules, two red mean Stop, stay and call for orders. Also: a

signal at a station that indicates by its position or by its color, that train orders are to be delivered to a

train, or that no orders are to be delivered.

Train Register

A book or form used at designated stations for registering time of arrival and departure of trains, and such

other information as may be prescribed.

Triangle

Additional track laid at a major junction to allow trains to be turned by running the three sides of the

triangle rather than reversing in a wye. Found outside major terminal stations where fixed passenger sets

need to be turned to equalize flange wear.

Triple Valve

An operating valve for charging the reservoir, applying the brake, and releasing the brake.

Truck Hunting

Rapid oscillation of an empty car truck at high speeds where the flanges tend to ride up on the head of the

rail.

TTC

Toronto Transit Commission, public transit system operating in Toronto, Ontario, Canada. First subway in

Canada (1954). Largest light rail system in North America.

Turnout Number

The ratio of the length of the tangent track to an equal unit of space between the tangent track and a point

on the branch track.

V

Unit

Another term for a locomotive engine.

Unit Train

A train composed entirely of one commodity, usually coal or mineral, and usually composed of cars of a

single owner and similar design, and usually destined for a single destination.

V

VAN

A term used in Canada for a caboose.

Variable Switch

A switch, designated by letter "V" or bowl painted yellow, when trailed through the switch points remain

lined in the position to which forced.

Varnish

Term used to refer to passenger trains, dating back to the late 19th century and the varnished passenger

coaches of the luxury trains such as those employed on the LV's Black Diamond and the C&O's Sportsman

W

Wandle

Affectionate rail slang for Wheeling and Lake Erie Railway which was successor to most Nickel Plate

Trackage.

Water Plug

The standpipe where a steam locomotive would stop to fill its tender with water.

Wheel Knocker

Another name for car knocker. This person would check the wheels for flaws.

Wheel Pull

Caused by the friction between the brake shoe and the wheel and transmitted to the rail.

Wheel Rolling

The wheel rotating on its axle theoretically without motion existing between the wheel and the rail at the

area of contact.

Wheel Slipping

The wheel rotating on its axle with motion existing between the wheel and rail at the area of contact.

Wheel Sliding

The wheel not rotating on its axle and motion existing between the wheel and rail at the area of contact.

Whistle Post

A specially marked post on the engineer's side of the train that tells him when to start whistling for a grade

crossing. Slower trains may delay whistling until closer to the crossing.

Windy

A slang term for a car going down a track with no air or hand brake applied

Wye

A track shaped like the letter "Y", but with a connector between the two arms of the "Y". A wye is used to

reverse the direction of trains or cars. A train pulls completely through one leg of wye, the switch is

thrown and reverses the direction, allowing the movement across the semi-loop track of the wye, and the

train is then headed in the opposite direction.

Y

Yard

A system of tracks, other than main tracks and sidings, used for making up trains, storing of cars and for

other purposes.

Yard Limits

A portion of main track designated by yard limit signs and by timetable, train order Form T or track

bulletin, which trains and engines may use as prescribed by Rule 93.

Yard Engine

An engine assigned to yard service.

Yellow Eye

A slang term for a yellow signal.